# Final Report

# Testbench Architecture

Below is the high level block diagram of tb_top.sv

# Test Plan Summary

## Test Plan Summary

### Overview

Our verification plan validates the functional correctness and AHB3-Lite protocol compliance of the memory slave using directed tests, corner-case scenarios, and constrained-random testing.

### Basic Transfer Verification

Single transfer operations are tested for byte, halfword, and word sizes in both read and write modes. These tests ensure correct byte-level write masking, proper data storage, and accurate readback without affecting adjacent memory locations.

### Burst and Address Generation

Burst operations including INCR4, INCR8, INCR16, WRAP4, and WRAP8 are verified. These tests confirm correct address incrementing, proper burst sequencing, and correct wrap-around behavior within defined boundaries without illegal address generation.

### Protocol Compliance and Control Behavior

Key AHB protocol behaviors are validated, including:

- IDLE transfers (no memory update)

- Back-to-back transfers (continuous operation without IDLE gaps)

- HREADY = 0 wait-state handling, ensuring stall and resume behavior preserves transfer integrity

### Read-After-Write (RAW) Behavior

Immediate read-after-write scenarios are tested to verify correct data visibility and ensure no contention or delay in updating memory values.

### Random Stress Testing

A constrained-random test of 10,000 transactions is executed to stress the design, uncover corner cases, and ensure robustness under unpredictable traffic conditions.

### Coverage and Completeness Goals

Functional and cross coverage ensures full protocol exploration:

- All possible HTRANS, HBURST, HSIZE, HWRITE values exercised

- Address space divided into bins for full memory coverage

- Coverage of HREADY stalls and HRESP error conditions

- Cross coverage of HBURST × HWRITE, HSIZE × HWRITE, HTRANS × HREADY

### Pass Criteria

- All directed tests pass

- Zero scoreboard mismatches

- Random testing completes without errors

- Full coverage closure achieved across all defined bins and crosses

# Assertion List

## a_write_read_correctness

Category: Functional

## a_no_change_without_write

Category: Functional

## a_latest_write_wins

Category: Functional

## a_word_write_atomic

Category: Functional

## a_no_mem_change_on_read

Category: Functional

## a_read_after_write

Category: Functional

## a_pipeline_rw_ordering

Category: Pipeline

## a_byte_write_targeted

Category: Masking

## a_byte_write_no_corrupt

Category: Masking

## a_hword_write_targeted

Category: Masking

## a_hword_write_no_corrupt

Category: Masking

## a_consec_reads_same_data

Category: Read Stability

# Bugs found

# Bug Description Caught By Assertion

Memory write not persistent a_write_read_correctness

Memory changes without write a_no_change_without_write

Read has side effects on memory a_no_mem_change_on_read

Back-to-back write ordering wrong a_latest_write_wins

Read-after-write returns stale data a_read_after_write

Byte lane targeting wrong a_byte_write_targeted

Byte write corrupts adjacent bytes a_byte_write_no_corrupt

Half-word targeting wrong a_hword_write_targeted

Half-word write corrupts other half-word a_hword_write_no_corrupt

Word write not atomic (partial update) a_word_write_atomic

Pipeline read-after-write ordering wrong a_pipeline_rw_ordering

Consecutive reads returning different data a_consec_reads_same_data

# Coverage Results

IMC RESULTS BEFORE MODIFICATIONS:

Total Coverage: 77.88%

IMC RESULTS AFTER MODIFICATIONS:

Total Coverage: 97.12%

- In hsize_cp, bins for 64-bits and above are ignored because the DUT is designed with a 32-bit data bus (HDATA_SIZE = 32). Therefore, transfers larger than 32 bits are not supported by the architecture and do not affect functional behavior.

- In hresp_cp, the HRESP_ERROR bin is ignored because the DUT implementation always generates HRESP_OKAY under all transaction conditions. Including this bin results in an unreachable coverage point.

- In haddr_cp, the higher address bins (above 1Kb) are ignored because the AHB master is constrained to generate addresses within a 1 KB range. As a result, the address is never generated outside this region.

- Ignoring bins from hsize_cp consequently reduces the  coverage space of cross_hsize_hwrite, and hence higher cross coverage.

# The reduced coverage in the cross_htrans_hready is because when HREADY is low, BUSY, NONSEQ, or SEQ cannot happen. Surprises and ImprovementsT